## 1. Kaiwu: A Multimodal Manipulation Dataset and Framework for Robot Learning and Human-Robot Interaction

Overview
This paper introduces Kaiwu, a large-scale multimodal dataset and collection framework designed to support robot learning, dexterous manipulation, and human–robot collaboration research. It targets real-world tasks with complex human interactions—a scenario where robots must infer intent and learn control policies from demonstrations.

Dataset & Data Modalities

- 20 human subjects interacting with 30 objects, producing 11,664 multimodal action segments.

- Modalities include:

  - Hand motion and operation forces, capturing dynamics of manipulation.

  - Multi-view RGB and depth video, enabling visual observation of tasks.

  - High-precision motion capture and skeletal tracking, essential for modeling human motion.

  - Eye gaze (first-person video) and electromyography (EMG), providing signals related to intent and muscle activity.

  - Audio recordings of assembly processes.

- All data streams are temporally synchronized with fine-grained annotations and segmentation labels to support multimodal learning and cross-modal fusion.

## 2. PoseTron: Enabling Close-Proximity Human-Robot Collaboration Through Multi-human Motion Prediction

Overview
This work proposes PoseTron, a transformer-based model for predicting human motion in close-proximity human–robot collaboration (HRC). It also introduces a new multimodal dataset (INTERACT) specifically focused on joint human–robot and human–human collaborative dynamics.

INTERACT Dataset (Context)
PoseTron is evaluated on INTERACT, a multimodal dataset that includes:

- 3D skeletal motion of multiple agents.

- RGB and depth (RGB+D) video streams.

- Gaze and robot joint state data.
This dataset supports studying rich agent dynamics in collaborative settings where robots and humans operate in the same workspace.

Model & Approach

- PoseTron uses a conditional attention mechanism in its transformer encoder to integrate motion patterns from all agents, effectively capturing interactive dynamics.

- Its decoder employs a multimodal attention mechanism to weight information from different modalities when predicting future motion trajectories.

- PoseTron achieves lower motion prediction error compared to baseline approaches widely used in HRC tasks.

## 3. Learning to Share Autonomy from Repeated Human-Robot Interaction

Overview
This work introduces SARI (Shared Autonomy across Repeated Interaction), an algorithm that enables assistive robots to learn shared autonomy policies from repeated human–robot interactions, without requiring a prior task definitions.

Core Contributions

- Task Recognition from Interaction Data – Embeds human behavior patterns to recognize the current task based on past interactions.

- Learning to Replicate Demonstrations – From repeated task demonstrations, the robot learns policies that mimic human guidance.

- Autonomy Arbitration – The system continually assesses confidence in its learned assistance, returning control to the human if uncertain.

- Theoretical Guarantees and Empirical Evaluation – Provides bounds on assistance error and shows robustness across various tasks through simulation and user studies.