# Incremental Learning Algorithms in Autonomous Driving and Robot Lifelong Skill Acquisition

## 1. Introduction

Autonomous systems, including self-driving vehicles and intelligent robotic agents, operate in dynamic, non-stationary environments where the distribution of data can change continuously over time. Traditional machine learning models are trained in a static setting, assuming access to all training data at once. This assumption is unrealistic for deployed systems, as new objects, tasks, and scenarios may emerge post-deployment. Re-training models from scratch for each update is computationally expensive, data-intensive, and often infeasible in safety-critical applications.

Incremental Learning (IL), also known as Continual Learning (CL), addresses this limitation by enabling models to learn sequentially from new data while retaining knowledge from previous tasks. A successful incremental learning system must balance:

- Plasticity: Ability to learn new tasks effectively

- Stability: Ability to retain knowledge from prior tasks

- Efficiency: Minimal memory, computation, and deployment overhead

- Safety and Robustness: Especially in autonomous driving, where errors can be catastrophic

In robotics, IL also enables lifelong skill acquisition, allowing robots to incrementally learn manipulation, navigation, and sensorimotor tasks without human supervision or reprogramming. This chapter surveys the major incremental learning algorithms applied in autonomous driving and robot lifelong skill acquisition, detailing:

- Algorithmic mechanisms

- Dataset preparation and design for incremental training

- Practical utility in autonomous driving and robotic systems

- Advantages, limitations, and research perspectives

## 2. Replay-Based Incremental Learning Methods

Replay methods mitigate forgetting by storing and reusing a subset of past experiences. They are widely used in perception-heavy systems, where replayed exemplars or synthetic data guide the model to retain prior knowledge.

### 2.1 iCaRL (Incremental Classifier and Representation Learning)

Mechanism:
iCaRL handles class-incremental learning by maintaining a fixed-size memory buffer of exemplars for previously learned classes. Training on a new task includes both new data and replayed exemplars. The model uses nearest-mean-of-exemplars (NME) classification, computing the mean feature vector for each class from stored exemplars, which stabilizes predictions as new classes are added.

Dataset Preparation:

- Sequentially add new object categories or tasks

- Select a fixed number of representative exemplars per class using a herding strategy

- Maintain memory size while balancing all classes

Autonomous Driving Utility:

- Incremental traffic sign recognition (e.g., newly deployed signs)

- Updating object detection modules for new vehicle types or road objects

- Adaptation to city-specific object distributions

Robotics Utility:

- Incremental object recognition for manipulation tasks

- Visual memory for grasping or assembly objects

Advantages:

- Strong performance on class-incremental tasks

- Explicit memory retention via exemplars

Limitations:

- Requires memory to store samples

- Exemplar selection can be computationally expensive

### 2.2 GDumb (Greedy DUMB Learner)

Mechanism:
GDumb is a surprisingly strong baseline that discards old model weights and maintains a balanced memory buffer of samples from all tasks. At each incremental step, the model is retrained from scratch using only this memory, avoiding catastrophic forgetting entirely by design.

Dataset Preparation:

- Maintain a fixed-size buffer

- Greedily select new samples while maintaining class balance

- No need to track parameter importance or task order

Autonomous Driving Utility:

- Offline retraining of perception modules

- Safe adaptation to updated object distributions

- Benchmarking and strong baseline for incremental perception studies

Robotics Utility:

- Consolidation of skill datasets in simulation

- Retraining policies from stored trajectories in episodic tasks

Advantages:

- Simplicity and strong empirical results

- Avoids forgetting without complex mechanisms

Limitations:

- Requires retraining from scratch

- Limited feasibility for online deployment

### 2.3 DER / DER++ (Dark Experience Replay)

Mechanism:
DER enhances classical replay by storing both input samples and their logits. During incremental learning, the loss combines classification loss for new data and distillation loss to match stored logits, preserving old knowledge. DER++ further incorporates ground-truth labels for replayed samples, improving stability.

Dataset Preparation:

- Store input–logit pairs for previous tasks

- Fixed memory buffer size

- Can complement new-task datasets with replayed examples

Autonomous Driving Utility:

- Incremental semantic segmentation across cities, lighting, or weather

- Continuous improvement of perception systems while maintaining old knowledge

Robotics Utility:

- Continual learning for sensorimotor mappings

- Retaining learned policies during incremental skill acquisition

Advantages:

- Combines replay with knowledge distillation

- Memory-efficient compared to full dataset storage

Limitations:

- Requires storage of logits

- Complexity higher than naive replay

## 3. Regularization-Based Incremental Learning Methods

Regularization-based methods avoid storing data by penalizing changes to important parameters, constraining the network to retain prior knowledge.

### 3.1 EWC (Elastic Weight Consolidation)

Mechanism:
EWC computes the Fisher Information Matrix to estimate the importance of each network parameter for previously learned tasks. The loss function for a new task includes a quadratic penalty for changing parameters with high importance.

Dataset Preparation:

- Sequential task datasets

- Parameter importance computed at task boundaries

- No replay buffer needed

Autonomous Driving Utility:

- Experimental incremental adaptation for perception

- Limited by highly overlapping and continuous task domains

Robotics Utility:

- Sequential reinforcement learning tasks

- Preserving previously learned locomotion or manipulation policies

Advantages:

- Memory-efficient

- No need to store old samples

Limitations:

- Poor scalability to large numbers of tasks

- Sensitive to hyperparameter tuning

### 3.2 SI (Synaptic Intelligence)

Mechanism:
SI tracks parameter contributions online by accumulating how much each parameter reduces the loss during training. Important parameters are penalized during updates for new tasks.

Dataset Preparation:

- Continuous stream of task data

- Online importance computation

- No task boundaries or labels required

Autonomous Driving Utility:

- Limited, mainly for controlled experiments

- Could be applied for incremental sensor adaptation

Robotics Utility:

- Online skill acquisition in continuous learning settings

- Memory-constrained embedded robots

Advantages:

- Task-boundary independent

- Lightweight memory footprint

Limitations:

- Less effective for complex perception tasks

### 3.3 LwF (Learning without Forgetting)

Mechanism:
LwF uses knowledge distillation from a previous model to preserve old knowledge. The current model is trained on new-task data while matching the teacher’s predictions for old tasks.

Dataset Preparation:

- Only new-task data required

- Teacher generates soft targets for distillation

Autonomous Driving Utility:

- Updating perception modules without access to old data

- Regression prevention in model updates

Robotics Utility:

- Policy updates without storing old trajectories

- Knowledge transfer between robot agents

Advantages:

- No data storage needed

- Flexible for various modalities

Limitations:

- Performance depends on teacher predictions

- Less robust than replay-based methods

### 3.4 MAS (Memory Aware Synapses)

Mechanism:
MAS estimates parameter importance by measuring sensitivity of output function to each parameter. Highly influential parameters are constrained during new task learning.

Dataset Preparation:

- Can use unlabeled data

- Online computation of parameter importance

Autonomous Driving Utility:

- Experimental applications for self-supervised perception adaptation

Robotics Utility:

- Continuous learning for manipulation and navigation

- Low-labeled data scenarios

Advantages:

- Can operate with unlabeled streams

- Suitable for online learning

Limitations:

- Requires careful sensitivity computation

- Less common in large-scale AD systems

## 4. Parameter Isolation and Architecture Growth Methods

These methods structurally prevent forgetting by isolating task-specific parameters or dynamically expanding the network.

### 4.1 PackNet

Mechanism:
PackNet prunes and freezes network weights after learning each task, reusing freed parameters for new tasks. This creates isolated subnetworks per task.

Dataset Preparation:

- Task-specific datasets

- Post-task pruning and masking

Autonomous Driving Utility:

- Rare due to latency and certification constraints

Robotics Utility:

- Modular skill libraries

- Safe incremental skill learning

Advantages:

- Zero forgetting by design

- Efficient reuse of parameters

Limitations:

- Pruning and masking overhead

- Memory allocation challenges

### 4.2 Progressive Neural Networks

Mechanism:
Each new task is assigned a new column in the network, previous columns are frozen, and lateral connections allow feature reuse.

Dataset Preparation:

- One dataset per task

- Column growth triggered by new tasks

Autonomous Driving Utility:

- Impractical due to unbounded growth

Robotics Utility:

- Sequential skill learning

- Zero-forgetting reinforcement learning

Advantages:

- Guaranteed retention

- Task-specific specialization

Limitations:

- Model growth becomes large

- Computational overhead

### 4.3 DEN (Dynamically Expandable Networks)

Mechanism:
DEN selectively expands the network only when existing parameters fail to represent new tasks adequately. Selective retraining balances reuse and expansion.

Dataset Preparation:

- Sequential task datasets

- Expansion triggered by performance threshold

Autonomous Driving Utility:

- Experimental studies for incremental perception

Robotics Utility:

- Long-horizon skill acquisition

- Adaptive model complexity

Advantages:

- Flexible capacity allocation

- Controlled forgetting

Limitations:

- Increased architecture complexity

- Sensitive to expansion criteria

## 5. Prompt and Adapter-Based Methods

### 5.1 Prompt Tuning

Mechanism:
Small task-specific prompt vectors are learned and prepended to inputs while the backbone is frozen. The prompts guide the model for new tasks.

Dataset Preparation:

- Task-specific datasets

- Pretrained backbone

Autonomous Driving Utility:

- Multi-modal scene understanding

- Vision-language perception updates

Robotics Utility:

- Instruction-based skill learning

- Multi-task robotic instruction generalization

### 5.2 Adapter Modules

Mechanism:
Small trainable layers inserted between frozen backbone layers. Each task updates only its adapters.

Dataset Preparation:

- Unified input format across tasks

- Task-specific adapter parameters

Autonomous Driving Utility:

- Incremental perception module updates

- Safe backbone freezing

Robotics Utility:

- Efficient multi-task skill acquisition

- Memory-efficient lifelong learning

## 6. Generative Replay Methods

### 6.1 Generative Replay

Mechanism:
Generative models (GANs, VAEs, diffusion models) synthesize previous task data for replay. The synthesized data is used alongside new-task data to prevent forgetting.

Dataset Preparation:

- Train generative model per task

- Use generated samples in incremental learning

Autonomous Driving Utility:

- Synthetic rare-event generation (e.g., accident scenarios)

- Safe data augmentation for perception

Robotics Utility:

- Simulation-based skill rehearsal

- Memory-efficient replay for long task sequences

## 7. Comparative Analysis (Scientific Metrics)

| Algorithm | Memory Cost | Computation Cost | Scalability | Utility in Autonomous Driving (AD) | Utility in Robot Lifelong Skill Acquisition |
| --- | --- | --- | --- | --- | --- |
| iCaRL | O(k·d) exemplar memory (typically 1–5% of full dataset; k exemplars per class, d feature dim) | O(N·d) per task (standard training + nearest-mean inference) | Moderate – degrades as class count grows | Incremental traffic sign, object, and lane-marking classification | Long-term object recognition for manipulation and navigation |
| GDumb | O(M) fixed buffer (M samples, usually ≤5–10% of dataset) | High: O(T·M) full retraining from scratch per task | Moderate – limited by retraining cost | Offline perception model refresh | Skill consolidation from curated experience |
| DER / DER++ | O(M·(x+z)) buffer storing inputs + logits/features | Medium – joint loss adds ~10–20% FLOPs | Good – stable with many tasks | Incremental semantic segmentation, detection | Policy/value retention in continual RL |
| EWC | O(P) Fisher diagonal (≈1× model parameters) | Low – quadratic penalty adds negligible FLOPs | Limited – assumes task separation | Experimental use in AD control policies | Sequential task learning in RL |
| SI (Synaptic Intelligence) | O(P) parameter importance traces | Low – lightweight online updates | Moderate – better than EWC in online settings | Experimental perception adaptation | Online motor skill acquisition |
| LwF | O(1) (no data storage, logits only at training time) | Medium – extra forward pass for distillation | Moderate – struggles with large domain shifts | Incremental perception (objects, drivable area) | Policy updates without replay |
| MAS | O(P) importance weights via gradients | Medium – gradient-based importance estimation | Moderate | Experimental AD perception/control | Manipulation & navigation skills |
| PackNet | O(P) masks per task (binary masks ~1 bit/param/task) | Medium – pruning + retraining | Moderate – capacity saturates | Limited applicability | Long-term skill libraries |
| Progressive Neural Networks | O(T·P) linear growth with tasks | High – parallel column inference | Poor – not scalable | Limited real-time feasibility | Skill accumulation without forgetting |
| DEN | O(P + ΔP) dynamic expansion (ΔP ≈ 5–20% per task) | High – expansion + regularization | Moderate | Experimental AD learning | Adaptive skill learning |
| Prompt Tuning | O(T·p) prompts (p ≪ P, usually <0.1% of model) | Low – backbone frozen | High – scales to many tasks | Multi-modal perception (VLM-based AD) | Instruction-based lifelong learning |
| Adapter Modules | O(T·a) adapters (a ≈ 1–5% of backbone) | Low – small extra layers | High | Incremental perception stacks | Multi-task robotic skills |
| Generative Replay | O(P_gen + M) generator + optional buffer | High – generation + training overhead | Moderate – generator limits scaling | Synthetic rare-event replay (accidents) | Simulation-based rehearsal of skills |

Where:

### Memory Cost Parameters

- k – Number of exemplars per class stored in memory (used in iCaRL).

- d – Feature dimensionality of the network (size of output from embedding layer).

- M – Total number of samples stored in a buffer (used in GDumb, DER, generative replay).

- x – Size of input sample (e.g., number of pixels × channels).

- z – Size of stored logits or feature representation per sample.

- P – Total number of network parameters (weights + biases).

- ΔP – Additional parameters added per task in expandable networks (DEN).

- T – Number of tasks learned sequentially.

- p – Size of task-specific prompt vectors (Prompt Tuning).

- a – Size of task-specific adapters (Adapter Modules).

- P_gen – Number of parameters in the generative model (GAN, VAE, diffusion).

### Computation Cost Parameters

- N – Number of new samples per task.

- d – Feature dimensionality (used in nearest-mean computation, iCaRL).

- FLOPs – Floating point operations; approximate measure of computation per forward/backward pass.

### Scalability Metrics

- Moderate/Good/Poor/High/Limited – Qualitative measure of how well the algorithm scales with increasing:

  - Number of tasks (T)

  - Model size (P)

  - Dataset size (N, M)

- This reflects both memory growth and computational overhead.

### Domain-Specific Terms

- AD – Autonomous Driving

- RL – Reinforcement Learning

- VLM – Vision-Language Model (used in prompt tuning)

### How to Interpret Formulas in Memory/Computation

- Memory Cost O(k·d) → iCaRL: storing k exemplars per class, each with feature vector of size d

- Memory Cost O(M·(x+z)) → DER: storing M samples, each with input (x) and logits/features (z)

- Memory Cost O(P) → EWC, SI, MAS: storing importance weights or Fisher diagonals for each network parameter

- Memory Cost O(T·P) → Progressive NN: network grows linearly with number of tasks T

- Computation Cost O(N·d) → iCaRL: cost proportional to number of new samples N and feature dimension d

- Computation Cost O(T·M) → GDumb: full retraining on M buffered samples for T tasks

## 8. Existing datasets

| Algorithm | Example Datasets | Domain & Relevance |
| --- | --- | --- |
| iCaRL | CIFAR-100, ImageNet-Subset, CORe50 | Used for class-incremental learning experiments; CORe50 is particularly for robotic object recognition in continual learning. Autonomous driving adaptation can use KITTI/BDD100K subsets for incremental object classes. |
| GDumb | CIFAR-100, TinyImageNet, CORe50 | Evaluates offline replay strategies; also used in robotic object recognition and incremental perception tasks. |
| DER / DER++ | CIFAR-100, ImageNet, CORe50, Cityscapes | Strong baseline for incremental segmentation (Cityscapes for AD perception) and classification (robotic objects in CORe50). |
| EWC | MNIST, PermutedMNIST, CIFAR-100, CARLA synthetic datasets | Classical RL/vision tasks for testing regularization-based forgetting prevention; CARLA provides AD-like perception scenarios. |
| SI (Synaptic Intelligence) | PermutedMNIST, CORe50, OpenAI Gym tasks | Used for online RL skill acquisition and continuous robotic learning. |
| LwF (Learning without Forgetting) | ImageNet, CORe50, KITTI/BDD100K | Incremental object classification; KITTI/BDD100K for perception module updates in autonomous driving. |
| MAS | CIFAR-100, CORe50, OpenAI Gym | Online skill acquisition, manipulation, and navigation in robotic settings. |
| PackNet | ImageNet, CIFAR-100, CORe50 | Multi-task classification; can be applied to robotic skill libraries. |
| Progressive Neural Networks | Atari-2600 RL Suite, OpenAI Gym, CORe50 | Sequential reinforcement learning; skill accumulation in robots and games. |
| DEN (Dynamically Expandable Networks) | CIFAR-100, ImageNet subsets, CORe50 | Adaptive task expansion for both classification and robotic skill acquisition. |
| Prompt Tuning | KITTI, BDD100K, nuScenes, Cityscapes, Robot Learning Datasets (RoboNet, RLBench) | Vision-language or multi-modal incremental learning for perception in AD and task instructions for robotics. |
| Adapter Modules | KITTI, BDD100K, Cityscapes, RoboNet | Incremental perception adaptation in autonomous driving and multi-task robotic skill learning. |
| Generative Replay | MNIST, CIFAR-100, Cityscapes, CARLA synthetic driving datasets, RLBench | Synthetic replay for rare events; used in simulation-to-real transfer in robotics and AD (e.g., accidents or unusual objects). |

### Notes on Datasets

- CIFAR-100, ImageNet, TinyImageNet: Standard benchmarks for incremental classification experiments; widely used to compare IL methods.

- CORe50: Designed for robotic object recognition with incremental object appearances and real-world variability.

- PermutedMNIST: Classic incremental learning benchmark for testing regularization-based methods.

- Cityscapes, KITTI, BDD100K, nuScenes: Autonomous driving datasets for object detection, semantic segmentation, and multi-task perception; can be split incrementally for IL experiments.

- CARLA synthetic datasets: Simulator-generated driving data for incremental perception and rare-event generation.

- OpenAI Gym / Atari RL Suite / RLBench: Used for incremental robotic skill learning, policy retention, and multi-task reinforcement learning.

- RoboNet: Multi-robot dataset for continual manipulation skill learning.

- RLBench: Benchmark for lifelong learning in robotic manipulation tasks.

## 9. Future Research Directions

- Integration of foundation models for lightweight incremental adaptation

- Generative replay combined with simulation-to-real skill transfer

- Safety and certification considerations in autonomous driving

- Evaluation benchmarks for long-term robotic skill acquisition

- Hybrid approaches combining replay, distillation, and adapters

Notes:

- see what others did

- real dataset not standard

- try to find existing projects that used ROS2 and incremental learning

- try to find innovative ideas about incremental learning algorithms